# CERTIFICATE OF CONFORMITY TO OEM REFERENCE

| Juratek Product Reference | MAN107 |
| --- | --- |
| Product Description | Brake Disc |
| OE Part Number | 81508030021,81508030041,81508030047,F465188,II37988 |
| Application(s): | MAN F90, F2000, L2000, M90, TGA, TGM, TGS and TGX series |
|  | Renault Premium |
|  | MAN – Bus |
| Supplier Quality Approval | BS EN ISO9001: 2008 |

We certify that the goods detailed above conform to the material & dimensional specifications required by Juratek Ltd and that they are fit for direct OEM replacement.

| Print Name: | Andrew Kibler |
| --- | --- |
| Signed: |  |
| Position: | Quality & Engineering Manager |
| Date: | 10 Feb 2012 |

With regard to the fitting of our MAN106, the required tolerance has been applied with a view to provide a good fit but still allow for the difficulty of manufacture. I would not recommend a drive fit because of the risk of failure in a Cast Iron material. It is not common practice to drive fit parts made of cast iron.

Our records show we have sold over 1500 units in the last 3 years without any recorded fitting problems.

It was stressed to me that there was a concern that because of the loose fitting, the weight and forces would be carried by the bolts/studs. The actual fitting of the disc to the hub relies on a very small length of the disc bore. A land of only a few millimetres. This would not be sufficient to withstand any torsion forces applied to the disc. It is the 12 bolts/studs that would carry these forces, which is why these are made from high tension steel.

Our materials are checked regularly to ensure they are to specification and the discs are measured on our CMM inspection machine for accuracy. I am more than happy that these discs overall meet ours and the customer requirements.

Yours Sincerely

Andrew Kibler

Quality & Engineering Manager

Juratek Ltd